ESP Error / Metrics Book Register

ESP32-C6 diag HTTPS event log. Transport: 32 KB bucket-v1, 16-byte records. Updated for PM-01…49, SD-01…19 (0x5B V4), ~10 s health, eFuse-corrected chip temp.

# 1. Families

| Family | Hex | Prefix | Role |
| --- | --- | --- | --- |
| PR | 0x10 | PR | Provisioning / unknown CMD |
| UA | 0x11 | UA | UART / STM link |
| SP | 0x12 | SP | SPI / audio |
| WF | 0x13 | WF | WiFi |
| TL | 0x14 | TL | DNS / TLS / AWS path |
| MQ | 0x15 | MQ | MQTT |
| ES | 0x16 | ES | System / heap thresholds |
| OT | 0x17 | OT | OTA |
| PM | 0x18 | PM | Continuous metrics ~10 s |
| SD | 0x19 | SD | STM 0x5B status chunks |

# 2. PM-49 chip temperature

PM-49 value = signed centi-°C (e.g. 3460 → 34.60 °C). IDF 5.1.6 stubs eFuse TSENS calib to 0; firmware reads ESP_EFUSE_TEMP_CALIB and applies corrected_C = raw_C − (deltaT/10) before storing PM-49.

# 3. Continuous PM list (summary)

| Code | Meaning |
| --- | --- |
| PM-01…03 | Heap free / min / largest |
| PM-04,21–23,41–48 | Per-task stack HWM |
| PM-05…28 | RSSI, reconnects, link flags, ping/DNS/TLS |
| PM-29…37 | SSID length + ASCII chunks |
| PM-38…40 | Heap total / used / % |
| PM-49 | ESP die temp centi-°C (eFuse-corrected) |

# 4. STM 0x5B V4 → SD-01…19

Wire: AA 4C 5B + 74-byte payload + CHK + 55. ESP accepts only LEN=0x4C. Store as 19 metric chunks; same event_time_s. Reassemble first 74 bytes. mcu_temp_c @71 (i16), mcu_temp_ok @73 — ignore temp if ok==0.

| Code | Bytes |
| --- | --- |
| SD-01…17 | [0..67] |
| SD-18 | [68..71] batt + mcu_temp_c lo |
| SD-19 | [72..73] mcu_temp_c hi + ok + pad |

# 5. Selected faults

| Code | Meaning |
| --- | --- |
| ES-09 | Free heap < 40 KB |
| ES-10 | Min heap < 32 KB |
| ES-11 | mqtt_process stack HWM < 512 B |
| TL-01…05 | DNS / TLS / AWS path faults |
| MQ-01…10, MQ-23 | MQTT connect / CONNACK / I/O / subscribe |
| OT-01…17 | OTA download / image / partition |
| WF-01…03 | WiFi disconnect / no AP / lost IP |
| UA-01/04/05 | UART error / re-init / STM silence |

# 6. Server checklist

Import CSV from DIAG_SERVER_REGISTRY.md (includes PM-49, SD-19).

Rebuild SD with 19 chunks / 74 bytes (not 16/63).

Decode PM-49 as signed centi-°C (already corrected on device).

Expect health metrics about every 10 s; flush poll remains 15 s.

Also see: DIAG_HTTPS_SERVER_API.md, COMPONENT_DIAGNOSTIC_ANALYSIS.md, changes_02_10_26.md.